ESP32 Smart Parking System

Technical Documentation

Version 1.0

Firmware Reference & Integration Guide

# 1. System Overview

The ESP32 Smart Parking System is an embedded firmware solution designed to automate vehicle entry and exit management in a parking facility. Built around the ESP32 microcontroller, it integrates ultrasonic distance sensing, servo-driven gate control, environmental hazard detection (flame and smoke), and ambient light-based LED control into a single cohesive system.

The system supports up to 20 parking slots and uses a Finite State Machine (FSM) to manage gate operation reliably, preventing concurrent entry/exit conflicts and ensuring accurate occupancy counts.

## 1.1 Key Features

- Dual-sensor vehicle detection for directional entry/exit tracking

- Servo-controlled gate with open/close state management

- Real-time occupancy counter (0 to 20 slots)

- Non-blocking flame and smoke alarm with variable buzzer frequency

- Automatic LED lighting based on ambient light level (LDR)

- FSM-based gate logic to prevent race conditions

- Serial debug output for monitoring and diagnostics

# 2. Hardware Configuration

## 2.1 Pin Assignments

| Pin | GPIO | Type | Description |
| --- | --- | --- | --- |
| FLAME_PIN | 34 | Analog Input | Flame/IR sensor — low value = fire detected |
| SMOKE_PIN | 35 | Analog Input | MQ-series smoke/gas sensor |
| BUZZER_PIN | 23 | Digital Output | Active buzzer for fire/smoke alarm |
| LDR_PIN | 32 | Analog Input | Light-dependent resistor for ambient light sensing |
| LED_PIN | 2 | Digital Output | Onboard/external LED for smart lighting |
| TRIG1 | 5 | Digital Output | Ultrasonic sensor 1 — trigger (entry side) |
| ECHO1 | 18 | Digital Input | Ultrasonic sensor 1 — echo (entry side) |
| TRIG2 | 17 | Digital Output | Ultrasonic sensor 2 — trigger (exit side) |
| ECHO2 | 16 | Digital Input | Ultrasonic sensor 2 — echo (exit side) |
| SERVO_PIN | 13 | PWM Output | Servo motor signal for gate control |

## 2.2 Configuration Constants

| Constant | Value | Description |
| --- | --- | --- |
| FLAME_THRESHOLD | 50 | Analog reading below this value triggers flame alarm |
| SMOKE_THRESHOLD | 3000 | Analog reading above this value triggers smoke alarm |
| LDR_THRESHOLD | 2500 | Above this value, LED turns ON (dark environment) |
| DETECTION_THRESHOLD | 15.0 | Distance in cm — object within range triggers sensor |
| TOTAL_SLOTS | 20 | Maximum vehicle capacity of the parking facility |
| SERVO_OPEN | 90 | Servo angle in degrees for open gate position |
| SERVO_CLOSE | 0 | Servo angle in degrees for closed gate position |

# 3. System Architecture

## 3.1 Main Loop Structure

The main loop() function runs continuously and executes the following stages on every iteration:

- Sensor Read — All analog and digital sensors are sampled (flame, smoke, LDR, two ultrasonic sensors)

- Smart Light Control — LED is driven based on LDR threshold comparison

- Buzzer Handling — Non-blocking alarm logic runs regardless of parking FSM state

- Parking FSM — Gate and occupancy logic processed via state machine

- Serial Debug — Sensor values and occupancy count printed at 115200 baud

A 50ms delay is applied at the end of each loop iteration for sensor stabilization.

## 3.2 Finite State Machine (FSM)

Gate operation is governed by a four-state FSM stored in the State enum. This prevents simultaneous entry and exit handling and ensures that the gate closes only after the vehicle has fully cleared both sensors.

| State | Description | Transition Condition |
| --- | --- | --- |
| IDLE | System waiting for vehicle approach | S1 triggered (entry) or S2 triggered (exit) |
| ENTRY_WAIT | Gate open, waiting for vehicle to reach S2 | S2 becomes active → WAIT_CLEAR |
| EXIT_WAIT | Gate open, waiting for vehicle to reach S1 | S1 becomes active → WAIT_CLEAR |
| WAIT_CLEAR | Vehicle crossing; waiting for both sensors free | Both S1 and S2 clear → update count → IDLE |

State Transition Logic

Entry flow (vehicle arriving):

IDLE: S1=true, S2=false, occupied < TOTAL_SLOTS  →  openGate(), ENTRY_WAIT

ENTRY_WAIT: S2=true                              →  WAIT_CLEAR

WAIT_CLEAR: S1=false, S2=false                   →  occupied++, closeGate(), IDLE

Exit flow (vehicle leaving):

IDLE: S2=true, S1=false, occupied > 0            →  openGate(), EXIT_WAIT

EXIT_WAIT: S1=true                               →  WAIT_CLEAR

WAIT_CLEAR: S1=false, S2=false                   →  occupied--, closeGate(), IDLE

# 4. Subsystem Details

## 4.1 Ultrasonic Distance Measurement

The getDistance() function drives a standard HC-SR04-compatible ultrasonic sensor using the following sequence:

- Pull TRIG LOW for 2 microseconds to reset

- Pulse TRIG HIGH for 10 microseconds to fire ultrasonic burst

- Measure ECHO pulse duration using pulseIn() with a 30ms timeout

- Convert duration to distance: distance (cm) = duration × 0.0343 / 2

If no echo is received within the timeout (object out of range or absent), the function returns 999 cm, which will not trigger detection. A return value below DETECTION_THRESHOLD (15 cm) is treated as a vehicle present.

## 4.2 Buzzer Alarm System

The handleBuzzer() function implements non-blocking alarm control using millis()-based timing, avoiding delay() calls that would stall the main loop.

| Condition | Interval | Behavior |
| --- | --- | --- |
| No hazard | — | Buzzer immediately silenced (digitalWrite LOW) |
| Flame OR Smoke | 500ms | Slow intermittent beep |
| Flame AND Smoke | 100ms | Rapid urgent beeping |

The buzzer state is toggled on each interval trigger, producing an alternating ON/OFF pattern. The lastBuzzerTime variable persists across calls to track elapsed time.

## 4.3 Smart Lighting

The LDR (Light Dependent Resistor) on GPIO 32 is sampled via analogRead(). When the reading exceeds LDR_THRESHOLD (2500), the environment is considered dark and LED_PIN (GPIO 2) is driven HIGH. This logic is a single line in the main loop and executes unconditionally on every iteration.

## 4.4 Gate Servo Control

The servo is managed through two helper functions that check the current gateOpen state before issuing a write command, preventing redundant servo movements:

openGate()  — if (!gateOpen): servo.write(SERVO_OPEN=90),  gateOpen = true

closeGate() — if (gateOpen):  servo.write(SERVO_CLOSE=0), gateOpen = false

The servo is initialized to the closed position in setup(). The ESP32Servo library is used for compatibility with ESP32's PWM timer system.

# 5. Data Flow & Serial Output

## 5.1 Serial Debug Format

The system outputs a debug line on every loop iteration at 115200 baud in the following format:

Flame:<value> Smoke:<value> LDR:<value> Occupied:<count>

Example output:

Flame:2048 Smoke:1200 LDR:3100 Occupied:5

All sensor raw ADC values (0–4095 for 12-bit ESP32 ADC) are printed alongside the current occupancy count. This is useful for threshold calibration during deployment.

## 5.2 Occupancy Bounds

After each FSM cycle, occupancy is clamped to safe bounds before the gate closes:

if (occupied < 0) occupied = 0;

if (occupied > TOTAL_SLOTS) occupied = TOTAL_SLOTS;

This prevents counter drift in edge cases such as power loss mid-transit or sensor glitches.

# 6. Dependencies & Setup

## 6.1 Required Libraries

| Library | Source | Purpose |
| --- | --- | --- |
| ESP32Servo | Arduino Library Manager | Servo control compatible with ESP32 PWM timers |

Install via Arduino IDE: Sketch → Include Library → Manage Libraries → search 'ESP32Servo'.

## 6.2 Board Configuration

- Board: ESP32 Dev Module (or equivalent)

- Upload Speed: 115200

- Flash Frequency: 80MHz

- Partition Scheme: Default

## 6.3 Initial Deployment

- Connect all sensors and servo per Section 2.1 pin assignments

- Open Serial Monitor at 115200 baud after flashing

- Verify flame and smoke readings are well below thresholds at rest

- Block each ultrasonic sensor at < 15cm to verify S1/S2 detection triggers

- Confirm servo moves to 90° on entry trigger and returns to 0° after vehicle clears

# 7. Customization & Calibration

## 7.1 Adjusting Thresholds

All detection thresholds are defined as constants at the top of the sketch for easy tuning. Observe raw ADC values on the Serial Monitor to set appropriate values for your hardware and environment:

- FLAME_THRESHOLD: Lower values = less sensitive. Raise if false alarms occur in bright sunlight.

- SMOKE_THRESHOLD: Higher values = less sensitive. Adjust based on MQ sensor baseline in clean air.

- LDR_THRESHOLD: Calibrate by observing LDR readings at your desired day/night switchover brightness.

- DETECTION_THRESHOLD: Set based on gate width. 15cm works for close-mount sensors; increase for wider lanes.

## 7.2 Changing Slot Capacity

Modify the TOTAL_SLOTS constant to match your facility's actual capacity:

const int TOTAL_SLOTS = 20;  // Change this value

## 7.3 Servo Angle Adjustment

If the gate does not fully open or close, adjust the servo angle constants:

const int SERVO_OPEN  = 90;  // Increase for wider swing

const int SERVO_CLOSE = 0;   // Decrease for negative direction close

# 8. Known Limitations

- No persistent storage — occupancy count resets to 0 on power cycle or reboot.

- Single-lane design — does not handle simultaneous entry and exit on separate lanes.

- No display output — occupancy must be monitored via Serial or external integration.

- Ultrasonic sensors can produce false readings in rain, fog, or with highly absorptive surfaces.

- Gate does not time out — if a vehicle stalls mid-transit, the FSM remains stuck in ENTRY_WAIT or EXIT_WAIT indefinitely until sensors clear.

# 9. Function Reference

| Function | Signature | Description |
| --- | --- | --- |
| getDistance | float getDistance(int trig, int echo) | Triggers ultrasonic pulse and returns distance in cm. Returns 999 on timeout. |
| handleBuzzer | void handleBuzzer(bool flame, bool smoke) | Non-blocking buzzer control. Beep rate varies by hazard severity. |
| openGate | void openGate() | Moves servo to SERVO_OPEN angle if gate is currently closed. |
| closeGate | void closeGate() | Moves servo to SERVO_CLOSE angle if gate is currently open. |
| resetSystem | void resetSystem() | Resets FSM to IDLE and clears entry/exit mode flags after gate closes. |

ESP32 Smart Parking System — Technical Documentation v1.0